TCF Acoustic Distress Engine

Engineering description — detection core v0.2

companion to core/ (Kotlin) and reference/distress_logic.py

# 1. Scope

A rule-based, on-device detector for a spoken call for help, combining a word-shape test (keyword proxy) with a paralinguistic distress score (tone). Target: an unattended, low-power unit in vehicles, later public spaces. The phone app is the data-collection and validation vehicle; the engine is written to port to C unchanged.

Design constraints that fixed the architecture:

- No training data yet. Works from first principles; every number is to be re-fitted from labelled recordings.

- Microcontroller-class target. ESP32 / Mindgrove class, no battery replacement. Rules out any transcription model on the device.

- Explainable decisions. Each alert traces to a few named quantities and their deviation from baseline.

- Tone is decisive. "Help, pass the salt" must not fire; a wordless scream must.

# 2. Architecture

Two independent questions are asked of every utterance and fused by a two-path rule:

- What is said — is the utterance shaped like "help" (/h ɛ l p/) or "bachao" (/b ə tʃ aː o/), and is it repeating?

- How it is said — does the voice carry the acoustic signature of fear (high arousal: raised, unstable pitch; high-frequency energy; roughness)?

audio (mono, any rate)

→ frames, 21.3 ms hop, ≈42.7 ms Hann window

→ frame features: level, f0, alpha ratio, Hammarberg, flatness, roughness proxy

→ voiced-event segmentation

→ per-event tone features + word-shape tests (help / bachao)

→ distress score: z-score fusion against calm baseline → logistic

→ decision: word path (3 word-shaped in 6 s, mean distress ≥ 0.50)

scream path (1 event ≥ 0.3 s, distress ≥ 0.85, roughness ≥ +2σ)

# 3. Framing

Frame sizes scale with the sample rate so the hop is always 21.3 ms and every frame-count rule keeps its meaning. Reference numbers at 48 kHz; the scaling rule is in Config.kt / framing_for().

| Quantity | 48 kHz | 16 kHz | Rule |
| --- | --- | --- | --- |
| Hop | 1024 (21.3 ms) | 341 (21.3 ms) | round(1024 · fs / 48000) |
| Window (NFFT) | 2048 (42.7 ms) | 1024 (64 ms) | next power of two ≥ 2048 · fs / 48000 |
| Pitch decimation | 3:1 → 16 kHz | 1:1 | round(fs / 16000) |
| Envelope block | 128 (2.7 ms) | 43 (2.7 ms) | round(128 · fs / 48000) |

# 4. Frame features

| Feature | Definition | Role |
| --- | --- | --- |
| level | RMS of the window, dBFS | loudness; voicing gate; floor tracking |
| f0 | Normalised autocorrelation on the ≈16 kHz copy, lags 60–1000 Hz; corr = Σxy / √(ΣxxΣyy); octave guard: shortest lag that is a local maximum within 92 % of the global maximum | pitch (semitones re 27.5 Hz downstream) |
| voiced | corr > 0.55 ∧ level > floor + 10 dB ∧ 60 ≤ f0 ≤ 1000 Hz | event segmentation |
| floor | noise floor, updated on unvoiced frames only: 0.995·floor + 0.005·level; init −60 dBFS | adaptive gate |
| alpha | 10·log10( E[1–5 kHz] / E[50–1000 Hz] ), dB | spectral tilt; rises with vocal effort |
| ham | Hammarberg index: peak dB in 0–2 kHz − peak dB in 2–5 kHz | falls with laryngeal tension |
| flat | spectral flatness 0.5–8 kHz (geometric / arithmetic mean of power) | ≈1 for /h/, /tʃ/ bursts; ≈0 for vowels |
| rough | Envelope at ≈375 Hz (RMS of 2.7 ms blocks), 128-point Hann spectrum of (env − μ)/μ; share of power in 30–150 Hz over total (bins ≥ 2). Gates: modulation depth < 5 % → 0; bins within ±2.5 bins of f0 and 2·f0 excluded | roughness proxy; keeps sub-harmonic (f0/2, f0/3) modulation, discards pitch pulses |

# 5. Event segmentation

- A voiced frame opens an event; unvoiced dips of ≤ 5 frames (107 ms: consonants, "help me") are absorbed; 6 consecutive unvoiced frames close it.

- 8 frames of pre-context are retained for the burst test; events with < 4 voiced frames (≈85 ms) are discarded.

- t_end = (start + frames + post) · hop. Audio from 8 frames + 50 ms before start to 100 ms after t_end is retained for export.

# 6. Word-shape tests

## 6.1 "help" — burst → vowel → stop

burst  = any pre-context frame: unvoiced ∧ flat > 0.25 ∧ level > floor + 6 dB

stop   = level(last voiced) − min(level of next 3 frames) ≥ 12 dB

durOk  = 0.1 s ≤ duration ≤ 3.0 s

helpScore = 0.45·burst + 0.30·durOk + 0.25·stop ;  help-shaped ⇔ helpScore ≥ 0.55

The vowel may be stretched to 3 s: "heeeelp" passes by construction. The consonant frame, not the timing, is what is tested.

## 6.2 "bachao" (बचाओ / বাঁচাও) — onset → mid-word burst → vowel

onset  = voiced run, 0.04–0.40 s

gap    = unvoiced run, 0.03–0.20 s, audible frames: mean flat ≥ 0.25 ∧ max alpha ≥ −5 dB (fricative energy)

core   = voiced run, 0.12–3.0 s

Detected inside one event (the gap was absorbed as an inner dip) or across two consecutive events, in which case the second absorbs the first in the repetition count. The burst between two voiced runs distinguishes it from the pre-voice /h/ burst. One rule covers Hindi and Bengali; the nasalised Bengali first vowel does not change the envelope. Both shapes count equally on the word path.

# 7. Per-event features and distress score

Over the voiced frames of an event: f0 mean and range (semitones), f0 slope (st/s, least squares), jitter (mean |Δf0| between frames, st), level mean and max, alpha, Hammarberg, roughness, duration.

zᵢ = clip( (xᵢ − μᵢ) / σᵢ , −3, +3 )

distress = sigmoid( 1.6 · ( Σ wᵢ·zᵢ / Σ|wᵢ| − 0.9 ) )        ∈ (0, 1)

| Feature | w | Baseline μ (σ) | Direction |
| --- | --- | --- | --- |
| rough | +1.0 | 0.18 (0.10) | rougher = more distress |
| f0 mean (st) | +0.8 | 33 (4) ≈ 185 Hz | higher |
| f0 range (st) | +0.6 | 4 (2.5) | wider |
| alpha (dB) | +0.6 | −14 (5) | more HF energy |
| Hammarberg (dB) | −0.5 | 26 (6) | lower |
| jitter (st) | +0.5 | 0.35 (0.25) | less stable |
| level (dBFS) | +0.4 | −30 (6) | louder |
| duration (s) | +0.3 | 0.35 (0.25) | longer |

Signs and relative magnitudes follow the affective-speech literature on fear (high arousal, low valence). The default baseline is a generic calm adult. All contributions are retained so an alert reads as, e.g., "pitch +2.1σ, roughness +1.8σ, level +1.2σ".

# 8. Decision

| Path | Condition | Outcome |
| --- | --- | --- |
| Scream | duration ≥ 0.3 s ∧ distress ≥ 0.85 ∧ z(rough) ≥ 2 | ALERT — single event, no word required |
| Word | ≥ 3 word-shaped events with t_end within 6 s ∧ mean distress ≥ 0.50 | ALERT |
| Word only | as above with mean distress < 0.50 | WORD_ONLY — logged and shown, no alarm |
| — | otherwise | STANDBY |

Evaluated once per closed event, scream path first. After an audible alert the input is blanked for 1 s so the alert tone is not analysed.

# 9. Calibration

6 s of the owner reading calmly. Frame-level mean and sd replace the baseline for f0, level, alpha, Hammarberg, roughness and jitter (with floors on σ: 2.5 st, 4 dB, 3 dB, 4 dB, 0.07, 0.15 st); f0 range μ = max(3, 2·sd(f0)); duration keeps the default. Persisted on the device; "default" is the fallback.

# 10. Cost and portability

- Per hop: one NFFT-point real FFT, one 128-point FFT, one autocorrelation over ≈680 lags on a 43 ms 16 kHz buffer. ≈2–3 % of one phone core at 48 kHz; real-time in fixed point on an ESP32-S3.

- No allocations in steady state beyond the per-event records. State: 12 s audio ring (export only), ≈1 s of frame history.

- Pure Kotlin, no dependencies; identical NumPy reference in Python. The two were run on ten synthetic cases at 48 kHz and 16 kHz: identical decisions, features within a few hundredths.

# 11. Known limitations

- Word-shape test is loose. Any burst-vowel-stop utterance passes; a tone with a sharp end passes on duration + stop. Discrimination currently rests on the repetition rule and the tone score. This is the component intended to be replaced by a trained keyword model.

- All thresholds hand-set. Nothing has been fitted to real recordings yet.

- No noise-robustness work. Vehicle cabin, engine harmonics, wind: untested. Only the floor tracker and the +10 dB voicing gate stand between them and the segmenter.

- Roughness is a proxy, not a Daniel–Weber / ECMA-418-2 roughness model.

- f0 tracking is minimal. Expect octave errors on breathy or very high voices; the f0-range feature is sensitive to them.

- One baseline per device. A passenger is scored against the owner's calibration or the generic default.

# 12. Relation to the Whisper-based prototype

Complementary, not competing. Whisper answers what was said with a real vocabulary and language identification; this engine answers how it was said plus a cheap word-shape gate. The exact string match on "help" in the Whisper build is its brittle point (no "heeeelp"; "help me" resets the count); the word-shape test here is the loose point. Running both on one stream and logging per utterance — Whisper text and token probability beside shape, distress and the z-vector — is the comparison of interest; CsvExport provides the row format.

Two items that would move the project:

- Grammar-constrained Whisper mode. whisper.cpp supports grammar-constrained decoding and the existing build links it (whisper_grammar symbols present, unused). Restricting output to help | bachao | ∅ and reading the token log-probability yields a keyword spotter with a confidence score, and a teacher for the small keyword model to be trained from synthetic (TTS + time-stretch) data for the microcontroller.

- Recordings. Calm, loud-but-calm and scared "help" / "bachao", several speakers, some in a moving vehicle. Either app can label and export them.

# 13. Deliverables

| Path | Content |
| --- | --- |
| core/Config.kt | every tunable number; rate scaling |
| core/Dsp.kt | FFT, semitone conversion, helpers |
| core/FrameAnalyzer.kt | frame features, roughness proxy |
| core/EventSegmenter.kt | segmentation, help / bachao shapes, per-event features |
| core/DistressScorer.kt | z-score fusion, calibration |
| core/DecisionEngine.kt | word path, scream path |
| core/DistressEngine.kt | pipeline entry point, audio ring |
| core/Export.kt | WAV and CSV writers |
| reference/distress_logic.py | reference implementation (NumPy only), identical logic |
| reference/testdata*/ | ten synthetic cases at 48 kHz and 16 kHz |
| INTEGRATION.md | API and wiring (three steps) |